Final Project Report: 2DOF Robotic Manipulator

January 6, 2026.

Ahmad Rashed Hazaimeh            ID: 162104

AMRO KHALED MOHAMMED ALJARRAH)         ID: 163904

### 1. Introduction

This project involves the design, modeling, and simulation of a 2-Degree-of-Freedom (2DOF) planar robotic manipulator3. The primary objective is to implement a control system that allows the robot's end-effector to precisely track a square-shaped trajectory within its workspace4.

### 2. Denavit-Hartenberg (DH) Parameters

Based on the physical dimensions provided (L1 = 0.2 and L2 = 0.15), the standard DH parameters were derived to define the robot's geometry:

| Link | ai​ (Length) | αi​ (Twist) | di​ (Offset) | θi​ (Angle) |
| --- | --- | --- | --- | --- |
| 1 | 0.2 | 0 | 0 | θ *1 |
| 2 | 0.15 | 0 | 0 | θ *2 |

### 3. Kinematics Analysis

- Forward Kinematics: Developed using the DH parameters to calculate the end-effector position (x, y) based on joint angles θ 1 and θ 2.

- Inverse Kinematics: Implemented using the MATLAB Robotics System Toolbox (Rigid Body Tree) to solve for the joint angles required to reach specific Cartesian coordinates.

Verification Results:

For the target point (0.15, 0.01)       θ 1 = -44.36                θ 2 = 133.58

### 4. Path Planning (Square Trajectory)

The manipulator was programmed to move sequentially between the following workspace coordinates to form a square:

Calculating Joint Angles for Square Path:

Point 1 (0.15, 0.01) -> Theta1: -44.36, Theta2: 131.68
Point 2 (0.25, 0.01) -> Theta1: -34.54, Theta2: 89.90

Point 3 (0.25, 0.11) -> Theta1: -8.79, Theta2: 78.37
Point 4 (0.15, 0.11) -> Theta1: -9.30, Theta2: 117.71
Point 5 (0.15, 0.01) -> Theta1: -44.36, Theta2: 131.68

### 5. Simulation Screenshots

- Simulink Model:

- Simscape Multibody Explorer:

- Command Window Output:

### 6. Real-Life Applications

- Automated Welding: Robots following precise geometric paths to join metal components in automotive manufacturing.

- 3D Printing (Additive Manufacturing): Moving an extruder head in a 2D plane to layer material and build complex parts.

- Laser/Waterjet Cutting: Navigating a cutting tool along programmed vectors to cut shapes out of flat sheets with high precision.

Thank you